彩色打印机电源

www.jensin.cn

JA750OKA

· 165 W 对流冷却

· 338 W 峰值

·省电模式

· 3.3V待机功能

·尺寸: L(285mm) x W(80mm) x H(47mm)

· IT 安全认证

· 5 年保修期

GREEN·POWER

规范

输入

常规

输入电压

输入频率

输入电流

浪涌电流

· 90-127VAC

· 47-63 Hz

· 5.0 A typical at 90 VAC, full load

· 30 A max at 140 VAC, cold start at 25 °C

节电模式

· DC output in 0.7W (3.3V/0.21A) power

Consumption 1.0W or less

DC output in 2W (3.3V/0.6A) power

Consumption 2.85W or less

输入保护               · Internal T6.3 A/250 V fuse in line

and neutral

输出

输出电压              · V1:3.3V;V2:5.0V;V3:24V

输出电流              · V1:1.5A;V2:6.3A;V3:6.8A

输出电压修正          · ±5%

启动延迟              · 1.5 s typical

启动上升时间          · 0.2 s MAX

保持时间              · 20 ms min at 100VAC

输出

隔离                  · 3300 VAC Input to Ground,

信号                  · Power save Inhibit, 3.3 V Standby

MTBF                 · 50000H or more in Ambient temperature 50°C

环境

工作环境               · 0 °C to +50 °C

冷却                   · Forced air cooling by FAN

工作湿度               ·10% to 85% RH, non-condensing

储存温度               · -10 °C to +60 °C

工作高度               · 3000 m

震动                   · Conditions for dropping impact:Drop hor izontally on

the concrete floor Height:50mm

EMC和安全

漂移

·

±0.2% after 20 min warm up

线路调节

负载调节

瞬态响应

波形和噪音

过压保护

过载保护

电路保护

·

·

·

·

±0.5% max

±2% V1, ±2% V2, ±2% V3

4% max. deviation, recovery to within

1% in 500 µs for a 50-75-50% load

change

V1andV2:250mVp-p, V3:500mVp-p20 MHz bandwidth

V1:4.5Vmax, V2:7.0Vmax, V3:32Vmax,

Latch off mode

·V1:3Amax, V2:15Amax, V3:13Amax

Latch off mode

·Latch off mode

爆发性噪音

高频连续传导

工频磁场

传导噪音

辐射噪音

谐波电流

·comply with JEITA IT-3001 and EN55024

Power supply line:±1.5KV

comply with JEITA IT-3001 and EN55024

Power supply line: 5V

comply with JEITA IT-3001 and EN55024

Power frequency magnetic field :3A/m

· FCC Class B,VCCI Class B,EN55022 Class B

· FCC Class B,VCCI Class B,and EN55022 Class B

·

.EN61000-3-2, JIS C 61000-3-2

·

AC DC

SHENZHEN CEC PANDA JENSIN TECHNOLOGY CO., LTD